Scheduler Requirements:

- Capabilities:

- Multiple Device Types (Geyser, Pool Pump, Borehole Pump)

- Each device has a unique address (set at factory)

- Each device gets name and serial number (done on site)

- Support for multiple devices of client (site inspection by technician)

- Hardware:

- 1 Ivy meter per device, communicating via Modbus RTU (RS-485) with RJ45 daisy chained cable for Multiple devices.

- Program Configuration:

- Each device has its own unique schedule

- Schedules use 24 hour format and can repeat on selected days ("MTWTFSS")

- Schedule configuration is in JSON format

- Define SoC% thresholds for load movement (turn on at upper & off at lower)

- Option to enable/disable LM algo on each device.

- Option to disable during loadshedding (Grid Off)

- Program Logic:

- Each device operates according to its configured time slots

- Device relay turns ON during scheduled periods and if SoC and LM conditions are met

- SoC thresholds control device operation based on battery state.

- Device state changes (ON/OFF) are triggered by schedule, SoC parameters and grid status

- Telemetry & Monitoring:

- Monitor each device telemetry (Granular and Critical): energy, power, relay state, LM state